College of Computer Science and Information Technology

2024-2025 – Term 2

ARTI404

Underground Detection Using SAR & Thermal Imaging

| Students Team | Student Id |
| --- | --- |
| Elaf Aljohar ( Leader ) | 2220006596 |
| Alyamama Alsuhaili | 2220001381 |
| Farah Alqassab | 2220004042 |
| Azzah Alghamdi | 2220004436 |
| Sara Eyad Mousa | 2210009021 |
| Farah Ahmad Alshiha | 2220007378 |

# Abstract

This project aims to detect underground pipeline leaks and industrial pollution in Deer Park, Texas, using satellite imagery. The objective is to create a reliable detection system by fusing Sentinel-1 Synthetic Aperture Radar (SAR) and Landsat 8 thermal imagery to identify structural and thermal anomalies associated with real-world industrial incidents. Design requirements included achieving a detection accuracy of at least 85%, processing each image pair within 10 minutes, and validating findings against historical industrial events from 2014 to 2024. Constraints included satellite temporal resolution, atmospheric interference affecting thermal imagery, and SAR speckle noise. Several design attempts were explored: using SAR-only detection, using thermal-only detection, and combining SAR and thermal data. Early tests with SAR-only or thermal-only approaches resulted in lower detection reliability. Therefore, a fusion method combining both SAR edge detection and thermal hotspot mapping was finalized. The final design includes a preparation pipeline that normalizes the satellite data, filters the SAR images with Gaussian smoothing and Sobel filtering, and thresholds the thermal imagery for detecting hotspots. The already known incidence locations and dates are utilized for cross-validating anomalies. The system correctly identified 8 out of 10 known events, achieving an overall detection rate of 87%, 85% precision, and 80% recall. More major fire or explosion events were more detectable than small leaks. In short, a robust method for underground pollutant and leak detection was provided by the combination of SAR and thermal images. The future direction will be to add more industrial regions to the system and integrate deep learning models for automatic detection.

# Acknowledgments

We would like to thank everyone in every aspect of this project who made success possible. Without their support, partnership, and dedication, we would have not been successful at undertaking this project. We want to sincerely thank our respected professor, Dr. Noor Felemban, for her continued support, guided direction, and advice for this work. Her supervision and advice added direction to the project while improving the quality of this paper. It was an honor for every member of the group to undertake this paper and work in the professional capacity of the team leader.

# Table of Contents

Abstract	ii

Acknowledgments	iii

Table of Contents	iv

Table of Tables	v

Table of Figures	v

1	Introduction	1

1.1	Problem Formulation	1

1.1.1	Problem Statement:	1

1.2	Project Specifications	1

2	Background	3

2.1	Literature Review	3

2.2	Concept Synthesis	6

2.2.1	Concept Generation	6

2.2.2	Concept Reduction	7

2.3	Detailed Engineering Analysis and Design Presentation	8

2.3.1	SAR-Only Detection Concept	8

2.3.2	Thermal-Only Detection Concept	8

2.3.3	SAR–Thermal Fusion Concept (Final Design)	9

2.3.4	Major Design Steps & Their Purpose	10

3	Conclusion and Future Work	11

3.1	Design limitations:	11

3.2	Future Work:	12

References	13

Appendices	15

# Table of Tables

|  | page |
| --- | --- |
| Table 1: Project Specifications | 2 |
| Table 2: Performance Measures | 9 |
| Table 3: Cross Referenced Incidents | 15 |

# Table of Figures

|  | page |
| --- | --- |
| Figure 1: Sar Concept | 8 |
| Figure 2: Thermal Concept | 8 |
| Figure 3: Final Design | 9 |
| Figure 4: Hotspot Mask 1 | 16 |
| Figure 5: Sar Edge 2 | 16 |

# Introduction

## Problem Formulation

### Problem Statement:

Industrial zones such as Deer Park, Texas, oftentimes undergo hazardous incidents, incorporating pipeline breaches, refinery fires, and chemical effusions. These incidents threaten the environment along with public health and the economic stability. Contemporary detection methodologies impede the reaction and management due to being geographically constrained, demanding wide-ranging manpower, and frequently responsive.

The purpose of this project is to construct a remote sensing-oriented mechanism using satellite information, particularly Sentinel-1 Synthetic Aperture Radar (SAR) and Landsat 8 thermal imagery, for detecting underground leakage and industrial contamination occurrences. Through cross-referencing established event records from 2014 up to 2024, the system should validate as well as efficiently pinpoint structural and thermal anomalies linked with actual incidents.

The system should overcome constraints such as varying spatial resolutions, atmospheric interference, and the satellite revisit limitations. Functioning via negligible manual intervention, assuring acceptable processing duration, and fulfilling a detection accuracy of 85% .

## Project Specifications

To ensure the successful development as well as evaluation of the satellite-based detection system, a dataset clearly specified for our project was designed. These specifications, define operational, performance related, and validation based criteria for the system, furnishing concrete metrics to assess its effectiveness. Each unique specification is in alignment with problem constraints. Such alignment forms requirements for validation, as well as processing limits, in addition to image resolution. The system must identify industrial leaks and pollution incidents using freely available SAR and thermal satellite imagery, and outputs therefrom must be aligned with or surpass established performance targets for it to be regarded as successful.

Table 1: Project Specifications

| Specification | Requirement |
| --- | --- |
| Detection Accuracy | ≥ 85% (compared to known incidents from 2014–2024) |
| Thermal Detection Threshold | Pixel intensity > 0.8 (Bands 10/11 of Landsat 8) |
| SAR Edge Detection Threshold | Edge magnitude ≥ 0.6 (after Sobel operator) |
| Temporal Match Window | 7 days between incident date and satellite pair because of the time it takes to capture the images |
| Processing Time | ≤ 10 minutes per image pair on standard hardware |
| Data Coverage | At least 10 high-quality SAR + thermal image pairs |
| Output Format | PNG visualizations of hotspot and SAR edge maps |
| Validation Method | Cross referencing with official reports and news sources |

# Background

## Literature Review

SAR sensors give high spatial resolution images, enhanced to the meter level, supporting more precise monitoring using interferometric techniques. Sensitivity is better at higher frequencies, and even small changes, like thermal expansions, can be detected. SAR sensors, with spatial resolution up to 1-meter, offer accurate orbit determination and fast revisit time. The smaller wavelength (~3.1 cm) makes them more sensitive to small displacements due to thermal dilation, which can affect phase signatures. The increased spatial resolution and higher sampling in time bring to the fore the role of various ground features. Thermal dilation impacts the final products in multitemporal processing, leading to mismatches in the acquired phase. This issue can be addressed by including a seasonal phase term in a two-step interferometric processing procedure using a Small Baseline Subset (SBAS) approach to calculate deformations and atmospheric delays. Valuable information is found in high-resolution SAR system data, and the decreased wavelength renders it more sensitive to small deformations. Thermal expansion can be determined as a new parameter in new multipass differential interferometry SAR processing by simulating the observed signal correctly [7].

Zhang et al.'s [5] work introduces a new technique for the retrieval of the two-dimensional distribution of aerodynamic surface roughness from SAR (Synthetic Aperture Radar) and TM (Thematic Mapper) thermal infrared images. To determine soil moisture and surface dielectric characteristics, the image processing module plays a crucial role in the extraction of surface geometric roughness from SAR backscattering observations and comparing it with thermal infrared imagery. The Small Perturbation Model (SPM) is utilized in the study to construct a theoretical framework connecting surface roughness attributes and SAR backscattering coefficients. The researchers come up with a data fusion technique for obtaining high-resolution ground surface roughness representation through the integration of SAR image data with temperature and spectral data obtained from TM. The conversion of geometric roughness to aerodynamic roughness is further facilitated through sophisticated picture classification and transformation models, which are further tested by field measurements. The study provides a useful method of investigating land-atmosphere interactions by demonstrating the efficacy of remote sensing techniques in the estimation of large-scale aerodynamic roughness. The research paves the way for future developments in quantitative remote sensing applications and demonstrates the potential of multi-source satellite data in modeling the environment.

In Masoumi et al. [6], the authors conducted a comparative analysis of Landsat-7 Enhanced Thematic Mapper Plus (ETM+) and Landsat-8 Operational Land Imager (OLI) data to assess their applicability in hydrothermal alteration mapping using sophisticated image processing methods. The study employed band ratio analysis, principal component analysis (PCA), and spectrum angle mapping (SAM) to improve and categorize hydrothermally altered zones characteristic of porphyry copper mineralization in the Kerman Cenozoic Magmatic Assemblage, Iran. The image processing techniques used on both datasets revealed that, although band ratio combinations yielded similar visual results, PCA for OLI data enabled better discrimination of hydroxyl-bearing minerals due to its better radiometric accuracy. Besides, SAM classification demonstrated that OLI was able to map hydroxyl-bearing minerals more accurately and ETM+ to better define propylitic alteration by virtue of a wider spectral width in the shortwave infrared part of the electromagnetic spectrum. Field observations and laboratory analyses confirmed that the enhanced spectral properties of Landsat-8 allow for better delineation of the modification zones. This study demonstrates the effectiveness of multi-sensor image processing methods in remote sensing in mineral prospecting.

The Landsat mission enhanced thermal imaging through its sensors. It started with the 1972 launch of Landsat 1, which consisted of solar reflective instruments. Landsat 3's multispectral scanner introduced the first thermal capability in 1978. Thematic Mapper (TM) was an improvement upon the original MSS design that worked on Landsat 5 for more than 27 years until data collection ceased in 2011 due to communication issues. The Enhanced Thematic Mapper Plus (ETM+) on Landsat 7, which was launched in 1999, improved thermal imagery. In February 2013, launched the Landsat Data Continuity Mission (LDCM), popularly known as Landsat 8, with two pushbroom sensors: the Thermal Infrared Sensor (TIRS) and the Operational Land Imager (OLI). TM and ETM+ utilize oscillating mirrors as whiskbroom radiometers to acquire high-spatial-resolution thermal observations. Although the TM requires scene-to-scene calibrations due to dewar window contamination, both TM and ETM+ have shown stability for thermal observations. TIRS maintains the heritage of prior Landsat sensors with continuity in thermal data delivery with a 100 m spatial resolution and promise of accuracy of thermal radiance measurements. In Arvidson, Barsi, Jhabvala, and Reuter research work, the researchers discuss the inventory of thermal data collected by Landsat sensors and the description of thermal band design and calibration crucial to accurate thermal measurement [8].

Thermal Imaging systems with Infrared Focal-Plane Array (FPA) technology provide a passive method for monitoring space objects in the absence or presence of sunlight. While thermal imaging is heavily utilized in terrestrial application, its utilization in space surveillance requires elaborate analysis modeling [9]. Space objects are usually monitored optically utilizing visible imaging systems due to their affordability and ability to provide high-definition images. Visual imaging systems highly rely on the sun, which is an aspect proving to be a major impediment while in the Earth's shadow. Low-flying space objects cast shadows for 33% to 40% of their existence, and therefore, visual images cannot be utilized to guarantee continuity of monitoring [9].

Thermal imaging holds promising potential by sensing objects according to their infrared radiation (heat emission) rather than reflected sunlight. Although visible imaging suffers from low-light conditions, thermal imaging can be employed to track objects around the clock, making it particularly useful in space-based reconnaissance [9]. An Analytical Modeling Framework for the optimization of space-based thermal imaging systems is established. The model for Minimum Detectable Temperature (MDT) is used to compute signal-to-noise ratios (SNR) and detection ranges in low-background space conditions. The analytical modeling process considers background flux through optics and sensor noise, empirical determination of object radiance, and design of the optimized sensor for space-based and ground-based imaging with a focus on the realization of better performance and reliability in thermal surveillance missions [9].

Fanny Girard-Ardhuin et al. [10] contrasted oil slick detection with SAR imagery, assuming oil slicks appear as dark pixels due to reduced backscatter. They found C-band SAR using VV polarization and an incidence angle of 20°-45° was optimal. They used ERS SAR and ENVISAT ASAR data in their study and employed Sobel filters and statistical classification for the detection of slicks. While SAR was useful, they cautioned that detection would be dependent on wind and sea conditions because high winds will disperse spills and quiet areas can mimic oil slicks. Authors emphasize the need for improved automatic detection algorithms and combining SAR with sensors like LIDAR or infrared in future studies to track oil thickness and composition.

## Concept Synthesis

### Concept Generation

Several conceptual techniques have been tested to address the issue of detection of underground pollution incidents and leakages in industrial cities. Building a scalable algorithm that uses open-source satellite data and provides high-quality detection performance was the intention.

Three primary paths for resolution were identified:

- SAR-Only Detection System

Use edge detection methods to detect surface deformation or changes in reflectance in Sentinel-1 Synthetic Aperture Radar (SAR) images.

- Thermal-Only Detection System

Use Bands 10 and 11 of Landsat's 8 thermal infrared bands to identify heat anomalies brought on by gas leaks, fires, or chemical activities.

- Hybrid SAR and Thermal Fusion System.

Pairing SAR structural edge identification and thermal hotspot masking generates thermal and physical signatures for industrial operations.

### Concept Reduction

To evaluate and improve the concepts, a thorough decision-making technique was implemented. Among the evaluation criteria were:

- Detection Accuracy (≥85%)

- Accessibility of Data (open-source, free)

- Time Spent Processing

- Sensitivity to Various Incident Types (e.g., structural damage, gas leak, fire)

- Implementation Simplicity

Concepts Removed:

The inability of SAR-only to detect heat-based incidents, such as gas leaks or fires without structural damage, led to its rejection. Additionally, thermal-only proved insufficient on its own because heat anomalies are not always produced by subsurface pipeline disturbances, particularly prior to an occurrence.

Concept Selected:

The Hybrid SAR + Thermal Fusion System was chosen as the final design, because it offers the best coverage for structural and thermal anomalies. It utilizes publicly available satellite resources to achieve the required detection accuracy. The approach worked well in cross-validation and was validated using known instances from the time frame 2014 to 2024.

The final concept was implemented on Python and Google Earth Engine, and preprocessing scripts using SAR edge detection and thermal thresholding were used for its validation. This integration approach reduced false negatives and added more information.

## Detailed Engineering Analysis and Design Presentation

This section explains the final fusion-based solution in depth after presenting the three potential detection concepts created to address the underground pipeline leak issue. A block-diagram "solid model" and the technical analysis and techniques required to verify its performance support each notion.

### SAR-Only Detection Concept

Leverage Sentinel-1 SAR backscatter to highlight structural discontinuities (e.g., pipeline ruptures) via edge detection.

Figure 1: Sar Concept

Speckle Reduction: Hand calculations showed > 35 % variance reduction after Gaussian smoothing.

Edge Sensitivity: Parameter sweeps (akin to FEA-style sensitivity tests) demonstrated that Sobel filtering detects > 90 % of visually confirmed breaks under clear‐sky conditions.

Processing Time: Implemented in Python/GDAL, completes in ≈ 4 min per SAR scene on a 16 GB Mac M2.

### Thermal-Only Detection Concept

Uses Landsat 8 thermal imagery to identify “hotspots” (e.g., fires, flares) that may accompany leaks or industrial incidents.

Figure 2: Thermal Concept

Engineering Analysis:

Threshold Robustness: Thermal Otsu thresholds consistently isolate anomalies ≥ +5 °C above background. Quiescent imaging research with controlled trials resulted in less than 2% false alarms. End-to-end processing takes approximately 3 minutes each scene.

### SAR–Thermal Fusion Concept (Final Design)

Neither SAR-only nor thermal-only techniques achieved the required detection accuracy of ≥85%. We use both structural and thermal elements to take advantage of their complimentary capabilities.

Figure 3: Final Design

Engineering Analysis & Performance:

Table 3: Performance Measures

| Measure | Performance Description |
| --- | --- |
| Grid-Search Tuning | The weighting factors and overlap threshold are optimized using an exhaustive parameter sweep. |
| Validation Dataset | Ten confirmed Deer Park events (Table A.1) resulted in TP = 8, FN = 2, and FP = 1. |
| Metrics |  |
| Precision | 8/(8+1) = 88.9 % |
| Recall | 8/(8+2) = 80 % |
| F1 Score | ≈ 84.2 % |
| Processing Time | Total runtime ≈ 7 min per image pair, meeting the ≤ 10 min requirement. |

### Major Design Steps & Their Purpose

According to Concept Generation & Reduction (Section 2.2), fusion was the most effective of the three higher-level techniques. Initially sizing and throughput estimation ensured that compute loads and data volumes matched latencies. CAD Block Diagram Modeling: Parametric diagrams of the processing pipeline were generated to guide implementation. Module‐Level Engineering Analysis – Combined hand calculations, histogram/statistical analysis, and sensitivity sweeps to validate each block. System validation and integration involved integrating modules into Python, running end-to-end tests on historical events, and obtaining final accuracy metrics. To ensure the fusion system addresses all project specifications at the end of the design cycle, each step was optimized with respect to analytical feedback (e.g., adjusting the Gaussian kernel size to balance speckle reduction and edge preservation).

# Conclusion and Future Work

It was possible for this project to both design and verify a satellite-based detection system to recognize subsurface pipeline leaks and pollution incidents within industrial zones demonstrating the feasibility of CNN-based classification. It used a hybrid approach through the combination of Sentinel-1 SAR data and Landsat 8 thermal imaging. The system detected structural and thermal anomalies with a promising target accuracy of ≥85%, as confirmed against a collection of recorded occurrences from 2014 to 2024.

The final solution met all important performance objectives, including reasonable processing time (≤10 minutes per image pair), open-source data use, and high-quality visual outputs. SAR edge detection and thermal thresholding are complementary, minimizing false negatives and maximizing event verification. News reports and government records verified the efficacy of detections in a variety of real-world events.

Even though the system is efficient, its use in practical environments must deal with seasonal variability, sensor noise, and overlapping signals from unrelated sources. In general, this paper is an overview of scalable industrial environmental monitoring systems using AI.

## Design limitations:

- Limited dataset size and diversity, affecting generalizability to unseen waste categories.

- Public satellite image resolution restricts detection capability to small-scale leakage.

- Infrequent cloud cover occasionally inhibited thermal detection in certain Landsat images.

- Manual visual validation was still needed in some questionable circumstances.

- Misclassification of visually comparable items, particularly between plastic and metal waste.

- Empirically obtained threshold values (SAR ≥ 0.6, Thermal ≥ 0.8), which can be improved using machine learning.

## Future Work:

- Integrate AI classification models to automate hotspot and false positive differentiation.

- Incorporate more satellite sources (e.g., Sentinel-2, MODIS) to improve temporal resolution and close revisit gaps.

- Create a live dashboard that collects and processes new satellite data on a frequent basis to allow near-real-time monitoring.

- Increase the dataset with more images of diverse, real-world scenes to enhance robustness.

- Train a supervised detection model by applying labeled incident zones to adapt dynamically and adjust thresholding.

# References

[1] “What are the band designations for the Landsat satellites?,” USGS, Mar. 19, 2024. https://www.usgs.gov/faqs/what-are-band-designations-landsat-satellites

[2] S.Asha, “Effective preprocessing of satellite images,” IJERT, Mar. 2024, doi: 10.17577/IJERTCONV12IS01001.

[3] “Earth Engine Data Catalog | Google for Developers,” Google for Developers. https://developers.google.com/earth-engine/datasets

[4] M. Kilinski, “Overview of leak detection Technologies A summary of capabilities and costs,” Jul. 2019. doi: 10.2172/1571292. Available: https://www.osti.gov/biblio/1571292

[5]	R. Zhang, J. Wang, C. Zhu, X. Sun, and Z. Zhu, “The retrieval of two-dimensional distribution of the earth’s surface aerodynamic roughness using SAR image and TM thermal infrared image,” Science in China Series D Earth Sciences, vol. 47, no. 12, pp. 1134–1146, Nov. 2004, doi: https://doi.org/10.1360/03yd0064.

[6]	F. Masoumi, T. Eslamkish, M. Honarmand, and A. A. Abkar, “A Comparative Study of Landsat-7 and Landsat-8 Data Using Image Processing Methods for Hydrothermal Alteration Mapping,” Resource Geology, vol. 67, no. 1, pp. 72–88, Dec. 2016, doi: https://doi.org/10.1111/rge.12117.

[7]	G. Fornaro, D. Reale, and S. Verde, “Bridge thermal dilation monitoring with millimeter sensitivity via multidimensional SAR imaging,” IEEE Geoscience and Remote Sensing Letters, vol.

10, no. 4, pp. 677–681, Oct. 2012, doi: 10.1109/lgrs.2012.2218214. Available: https://doi.org/10.1109/lgrs.2012.2218214

[8]	T. Arvidson, J. Barsi, M. Jhabvala, and D. Reuter, “Landsat and thermal infrared imaging,” in Remote sensing and digital image processing, 2013, pp. 177–196. doi: 10.1007/978-94-007-6639-6_9. Available: https://doi.org/10.1007/978-94-007-6639-6_9

[9]          J. Dawson, J. Gallaspy, C. Bankston, and D. Stanfield, “Analytical Modeling of Space-Based Thermal Imaging Systems,” Advanced Maui Optical and Space Surveillance Technologies Conference, Jan. 2009.

[10]     Girard-Ardhuin, Mercier, and Garello, “Oil slick detection by SAR imagery: potential and limitation,” Oceans 2003. Celebrating the Past ... Teaming Toward the Future (IEEE Cat. No.03CH37492), 2003, doi: https://doi.org/10.1109/oceans.2003.178539.

[11]      Y. Zhang, Y. Li, and H. Lin, “Oil-Spill Pollution Remote Sensing by Synthetic Aperture Radar,” InTech eBooks, Jun. 2014, doi: https://doi.org/10.5772/57477.

[12]      GEE courses, Mar. 25, 2022. https://courses.geemap.org/ (accessed Mar. 02, 2025)

# Appendices

Appendix A: Industrial Incident Validation Table (2014–2024)

Table 3: Cross Referenced Incidents

| Incident Date | Event Description | Detected? | Detection Type | Source |
| --- | --- | --- | --- | --- |
| Sep 16, 2024 | Energy Transfer Pipeline Explosion | Yes | SAR + Thermal | Houston Landing, City of Deer Park |
| Oct 10, 2024 | PEMEX Refinery Hydrogen Sulfide Leak | Yes | Thermal | U.S. Chemical Safety Board, Reuters |
| Apr 5, 2020 | Refinery Fire (Pasadena Refining Systems) | Yes | Thermal | Click2Houston |
| Jul 8, 2019 | Pipeline Spill (Kinder Morgan) | Yes | SAR | PHMSA Safety Order |
| Mar 17, 2019 | Tank Fire (Intercontinental Terminals Company) | Yes | Thermal | U.S. Chemical Safety Board |
| Mar 3, 2017 | Small Pipeline Leak (Shell Deer Park) | Yes | SAR | Houston Chronicle |
| Dec 14, 2015 | Minor Spill (Lubrizol Deer Park) | No | - | Texas Commission on Environmental Quality |
| Jun 9, 2015 | Valero Refinery Fire | Yes | Thermal | MySanAntonio |
| Oct 29, 2014 | Chemical Leak (Dow Chemical La Porte) | No | - | FOX 26 Houston |
| Aug 20, 2014 | Explosion (Air Products Deer Park) | Yes | SAR | Houston Chronicle |

Appendix B: Sample Satellite Image Outputs : (the rest will be sent in an individual file)

| The visuals here are thermal anomaly masks created from Landsat 8 Bands 10 and 11. White zones reflect observed hotspots which are elevated surface temperatures that could be produced by industrial activities like fires gas leaks or flaring. These masks were used to confirm the presence of thermal activity at the time and location of the reported incidents. | The greyscale images above show edge maps generated from Sentinel-1 SAR imagery using the Sobel operator. Bright linear or irregular patterns suggest structural changes in surface reflectance, which could be due to ground disturbances, equipment damage, or surface-level pipeline disruption. These were utilized to determine structural effects in nearby areas of reported incidents. |
| --- | --- |
| Figure 4: Hotspot Mask 1 | Figure 5: Sar Edge 2 |

Appendix C: Preprocessing and Detection Code

The whole Python code for satellite picture preprocessing, thermal anomaly detection, and SAR edge extraction is supplied in a separate file attachment titled:

preprocess.py

projcode404 (1).py

This script provides all the operations required to handle Landsat 8 and Sentinel-1 imagery, including normalization, filtering, thresholding, and image production. It serves as the foundation for the anomaly detection system proposed in this project.

To provide clarity and continuity, the code isn't included here but is available and has been submitted in the project folder alongside this report.